# Document S1– Shopping & Machining List

| Item ID | Source | Accessibility |  | Description | Link | Image | Quantity |
| --- | --- | --- | --- | --- | --- | --- | --- |
| General tools |  |  |  |  |  |  |  |  |
|  | Thorlabs |  | purchase | Spanner Wrench 1 | SPW30 |  | 1 |
|  | Thorlabs |  | Purchase | Fiber stripping tool | FTS4 |  | 1 |
|  | Thorlabs |  | purchase | Fiber Cleaver | XL411 |  | 1 |
|  | Thorlabs |  | purchase | Spanner Wrench 2 | SPW602 |  | 1 |
|  | Thorlabs |  | purchase | UV Curing LED System | CD20K2 |  | 1 |
|  | Thorlabs |  | purchase | Handheld laser source (630nm) | HLS635 |  | 1 |
|  | Thorlabs |  | purchase | Power and Energy meter | PM100D |  | 1 |
|  | Thorlabs |  | purchase | Green LED | M530F2 |  | 1 |
|  | Thorlabs |  | purchase | NIR Detector Card | VRC4 |  | 1 |
| Core optics module |  |  |  |  |  |  |  |  |
| Mechanical components |  |  |  |  |  |  |  |  |
|  | Thorlabs |  | purchase | Nexus Breadboard | B3045L |  | 1 |
|  | Thorlabs |  | purchase | Sorbothane Feet | AV6/M |  | 1 |
|  | Thorlabs |  | purchase | Fiber Adapter Plate | SM1SMA |  | 1 |
|  | Thorlabs |  | purchase | Adjustable Mirror Mount | POLARIS-K05S2 |  | 2 |
|  | Thorlabs |  | purchase | Rotation Mount | MRM05/M |  | 1 |
|  | Thorlabs |  | purchase | M6 cap screws kit | HW-KIT2/M |  | >20 |
|  | Thorlabs |  | purchase | Ø900 µm Hytrel Furcation Tubing | FT900Y |  | 3 |
|  | Thorlabs |  | purchase | 3 axis Microblock stage | MBT616D/M |  | 1 |
|  | Thorlabs |  | Purchase | 3-Axis MicroBlock Stage | MBT602/M |  | 1 |
|  | Thorlabs |  | Purchase | Compatible Flexure Stage Mount | HCS020 |  | 1 |
|  | Thorlabs |  | Purchase | Fiber Clamp Holds fiber | HFF001 |  | 3 |
|  | Thorlabs |  | purchase | Fixed mounting bracket | AMA007/M |  | 1 |
|  | Thorlabs |  | purchase | SM05 Threaded Adapter to laser source | AD1109F |  | 1 |
|  | Thorlabs |  | purchase | Kinematic Cage cube | DFM1T3 |  | 1 |
|  | Thorlabs |  | purchase | Kinematic cage cube base | DFM1B |  | 1 |
|  | Thorlabs |  | purchase | Lens Tube | SM1L05 |  | 3 |
|  | Thorlabs |  | purchase | End Cap for Machining | SM1CP2M |  | 1 |
|  | Thorlabs |  | purchase | Coupler | SM1T2 |  | 3 |
|  | Thorlabs |  | purchase | Optical Beam shutter with controller for PMT | SHB1 |  | 1 |
|  | Thorlabs |  | purchase | Post mounting adapter | SHM1/M |  | 1 |
|  | Kavli NTNU |  | Self-made | Coupling Box | Github(1) |  | 1 |
|  | Kavli NTNU |  | Self-made | Box cup | Github(1) |  | 1 |
|  | Kavli NTNU |  | Self-made | Coupling holder | Github(1) |  | 1 |
|  | Kavli NTNU |  | Self-made | Coupling Protector | Github(1) |  | 1 |
|  | Kavli NTNU |  | Self-made | Collimator Assemble tool | Github(1) |  | 1 |
|  | Kavli NTNU |  | Self-made | Collimator Holder | Github(1) |  | 1 |
|  | Kavli NTNU |  | Self-Made | Control box shell | Github(1) |  | 1 |
|  | Kavli NTNU |  | Self-Made | Control box cup | Github(1) |  | 1 |
| Optical components |  |  |  |  |  |  |  |  |
|  | Toptica |  | purchase | Laser source | FemtoFiber ultra 920 |  | 1 |
|  | Schott |  | purchase | Tapered fiber bundle (TFB) | C54706 | NA | 1 |
|  | NKT Photonics |  | purchase | Hollow-core PCF, HC-920 | K50-060-00 | NA | 1 |
|  | Sunlight |  | purchase | Glass rods (ZF62) | GLA-10x150-AR800-1100 | NA | 3 |
|  | Sunlight |  | purchase | Glass flange ∅0.126x6.5mm | TUB-1.8x6.5-0.127 | NA | 2 |
|  | Thorlabs |  | purchase | Prisms direct light from the seed laser up to the HC-920 fiber. | MRA12-P01 |  | 8 |
|  | Thorlabs |  | purchase | Emission filter 525 nm green channel | MF525-39 |  | 1 |
|  | Thorlabs |  | purchase | Emission filter 630 nm red channel | MF630-69 |  | 1 |
|  | Thorlabs |  | purchase | Shortpass Filter | FESH0750 |  | 2 |
|  | Thorlabs |  | purchase | Dichroic Mirror | DMLP567R |  | 1 |
|  | Thorlabs |  | purchase | Aspheric Condenser Lens | ACL25416U-A |  | 3 |
|  | Thorlabs |  | purchase | Coupling lens | C230TMD-B |  | 1 |
|  | Thorlabs |  | purchase | Half-Wave Plate | WPHSM05-915 |  | 1 |
|  | Thorlabs |  | purchase | Protected Silver Mirrors | PF05-03-P01 |  | 2 |
|  | Edmunds |  | Purchase | Collimating Lens | #84-128 |  | 1 |
| Electrical components |  |  |  |  |  |  |  |  |
|  | Mirrorcle Technologies |  | Purchase | MEMS driver (controller) BDQ PicoAmp 5.4 B160 | DR-11-054-00 |  | 1 |
|  | Digikey |  | Purchase | BNC cables | CCBNS-MM-RG174-36 |  | 8m |
|  | RS |  | purchase | DSUB15 connector plug (male) | 472-859 |  | 2 |
|  | RS |  | purchase | Backshell | 765-9448 |  | 2 |
|  | RS |  |  | DSUB15 Connector Socket (female) | 472-865 |  | 2 |
|  | Digikey |  | purchase | 6-pin connector for MEMS | FH19C-6S-0.5SH(10) |  | 1 |
|  | Industrifil |  | purchase | Single wire cables | link |  | 6 |
|  | Thorlabs |  | purchase | K-Cube Piezo Controller | KPZ101 |  | 1 |
|  | Thorlabs |  | purchase | PMTs | PMT2101/M |  | 2 |
|  | Thorlabs |  | purchase | Controller for shutter | SHB1T |  | 1 |
| Scope mounting module |  |  |  |  |  |  |  |  |
| Mechanical components |  |  |  |  |  |  |  |  |
|  | Thorlabs |  | Purchase | Aluminum optical breadboard | B3060A |  | 1 |
|  | Thorlabs |  | Purchase | Sorbothane Feet Ø45.0 mm, Internal M6 Mounting Thread, 4 Pieces to support previous breadboard. | AV6/M |  | 1 |
|  | Thorlabs |  | Purchase | One-sided Construction Rail, Clear Anodized, 95x500 mm | XT95SP-500 |  | 2 |
|  | Thorlabs |  | purchase | Rail carriage suitable for sliding onto previous rail and with M6 tapped holes | XT95RC4/M |  | 2 |
|  | Thorlabs |  | Purchase | Mounting Post Ø1.5" L=350mm | P350/M |  | 1 |
|  | Thorlabs |  | Purchase | Post Mounting Clamp | C1545/M |  | 1 |
|  | Thorlabs |  | purchase | Manual Rotation Stage | RP03/M |  | 1 |
|  | Thorlabs |  | Purchase | Studded Pedestal Base Adapter | BE1/M |  | 1 |
|  | Thorlabs |  | purchase | Optical Post | TR75/M |  | 2 |
|  | Thorlabs |  | Modify from product | Running wheel hardboard | Github, TB4 |  | 1/4 |
|  | Thorlabs |  | purchase | M6 spacers & washers | NA |  | >4 |
|  | Thorlabs |  | purchase | Spacer on both sides of wheel | PS1M |  | 2 |
|  | SKF |  | purchase | Bearing OD 19mm ID 6mm | 626-2Z |  | 1 |
|  | Thorlabs |  | purchase | Right-angle Clamp | RSA90/M |  | 1 |
|  | Thorlabs |  | purchase | Pillar posts | RS50/M |  | 2 |
|  | Thorlabs |  | purchase | Universal Post holder | UPH100/M |  | 3 |
|  | Thorlabs |  | purchase | Locking Ball and Socket Mount; to support LEDs near Tracking Camera | TRB1/M |  | 8 |
|  | Thorlabs |  | purchase | Adapter camera-lens | SM1A10Z |  | 2 |
|  | Kavli NTNU |  | Self-made | MINI2P Holder P1 | Github(1) |  | 1 |
|  | Kavli NTNU |  | Self-made | MINI2P Holder P2 | Github(1) |  | 1 |
|  | Kavli NTNU |  | Self-made | MINI2P Holder P3 | Github(1) |  | 1 |
|  | Kavli NTNU |  | Self-made | MINI2P Holder P4 | Github(1) |  | 1 |
|  | Kavli NTNU |  | Self-made | Wheel Holder | Github |  | 1 |
|  | Kavli NTNU |  | Self-made | Headbar holder | Github |  | 1 |
| Optical and electrical components |  |  |  |  |  |  |  |  |
|  | Thorlabs |  | purchase | 850nm IR LED Array Light Source | LIU850A |  | 1 |
|  | Thorlabs |  | purchase | CMOS Sensor camera | CS165MU/M |  | 1 |
|  | Edmund Optics |  | purchase | Lens focal length 4.5mm Or 8.5mm for lateral and frontal camera to control head fixation | F1.3 f8.5 2/3” |  | 1 |
|  | Edmund Optics |  | purchase | Basler camera for Animal Tracking (top or bottom view) | acA2040-90um |  | 1 |
|  | Physik Instrumente (PI) |  | purchase | DC Motors | M-112.2DG |  | 3 |
| Mobile cart and controlling system |  |  |  |  |  |  |  |  |
| Mechanical and electrical components |  |  |  |  |  |  |  |  |
|  | Thorlabs |  | Purchase | Mobile Cart | POC001 |  | 1 |
|  | Thorlabs |  | Purchase | Optical breadboard 600x900x55mm | PBG52506 |  | 1 |
|  | Thorlabs |  | Purchase | Optional drawer | POD001 |  | 1 |
|  | Schroff |  | Purchase | 19-inch rack | Ref 721-2708 |  | 1 |
|  | McMASTER-CARR |  | Purchase | Span-in Nuts | 90680A729 |  | 10 |
|  | Dell |  | Purchase | Workstation is an Intel core i9 with operating windows 10 Pro | 7080 |  | 1 |
|  | Dell |  | Purchase | Monitors | NA | NA | 1 |
|  | Vidrio Technologies LLC |  | Purchase | vDAQ card provides data acquisition and control of Laser, TLens, shutter, among others | V-vDAQ.R1 |  | 1 |
|  | Vidrio Technologies LLC |  | Purchase | vDAQ breadboard | V-vDAQ.R1 |  | 1 |
|  | Physik Instrumente (PI) |  | Purchase | Motion Controller for PI motors | C-884.4DC |  | 1 |
|  | Toptica |  | Purchase | Controller for 920nm laser | Toptica |  | 1 |
|  | Dustin |  | Purchase | USB-hub ports | Deltaco |  | 1 |
|  | Thorlabs |  | Purchase | BNC Male to BNC Male | CA3136 |  | 8 |
|  | Thorlabs |  | Purchase | BNC to SMA Male Connector | CA2848 |  | 2 |
|  | Thorlabs |  | Purchase | SMC connector | PAA101 |  | 2 |
|  | Thorlabs |  | Purchase | SMA-to- SMA cable | CA2912 |  | 2 |
|  | Thorlabs |  | Purchase | Power supply for Tlens driver | TPS002 |  | 1 |
| Software |  |  |  |  |  |  |  |  |
|  | Vidrio Technologies LLC |  | Purchase | Open-source software for the whole system | ScanImage 2020 | NA | 1 |
|  | Others |  | Free or purchase | See Document S4 | NA | NA | 1 |
| MINI2P miniscope |  |  |  |  |  |  |  |  |
| Mechanical components |  |  |  |  |  |  |  |  |
|  | Kavli NTNU |  | Self-made | Scope Body P1 | GitHub |  | 1 |
|  | Kavli NTNU |  | Self-made | Scope Body P2 | GitHub |  | 1 |
|  | Kavli NTNU |  | Self-made | Scope Body P3 | GitHub |  | 1 |
|  | Kavli NTNU |  | Self-made | Stitching Adapter | GitHub |  |  |
|  | Kavli NTNU |  | Self-made | Baseplate | GitHub |  | 10 |
|  | Kavli NTNU |  | Self-made | Alignment Tool | GitHub |  | 1 |
|  | TR fastenings |  | Purchase | Screws for MINI2P, M1.4; M1.6; M2.0; | NA |  | >10 |
| Optical components |  |  |  |  |  |  |  |  |
|  | Domilight |  | Purchase | Scan Lens | D0166 |  |  |
|  | Sunlight |  | Purchase | Dichroic Mirror | DMSP0405 |  |  |
|  | Domilight |  | Purchase | Objective1 | D0213-3X |  | 1 |
|  | Domilight |  | Purchase | Objective2 | D0254-3X |  | 1 |
|  | Domilight |  | Purchase | Objective3 | D0277-3X |  | 1 |
|  | Polight |  | Purchase | μTLENS | TLENS-NIR-D-45 |  | 1 |
| Electrical components |  |  |  |  |  |  |  |  |
|  | Mirrorcle |  | Purchase | MEMS “fast” 5600 Hz | Fast - A7M10.2-1000AL |  | 1 |
|  | Mirrorcle |  | Purchase | MEMS “slow” 2000 Hz | A3I12.2-1200AL-LCC20-TW |  | 1 |
|  | Kavli NTNU |  | Purchase | MEMS PCB |  |  | 1 |